# Agibot A2 Ultra General Documentation

Comtrade System Integration – AI/Robotics Team

Contents

Section I – Introduction:	3

Section II – Hardware:	3

PC 1:	4

PC 2:	4

A2 – Cameras and LiDAR:	4

Warnings and Notices:	5

Section III – Software:	5

Software – Supervisor:	6

Software – Agibot AimDK:	8

Section IV – Locomotion and Motion Controls:	9

General Information:	9

How to use Locomotion via AimDK:	10

How to use Motion Controls via AimDK:	10

How Motion Controls work:	11

Safety and Dangers:	12

Section V – Emergency and Safety Features:	12

E-Stop:	12

LiDAR Safety:	13

Supervisor Safety Features:	13

# Section I – Introduction:

The Agibot A2 Ultra is a humanoid robot built by the Agibot Corporation. It features several high-end embedded devices (PCs) which allow it to perform a wide variety of functions.

This humanoid robot is 169cm tall and about 70kg. It has 3 chest cameras (2 fisheye, 1 central RGB), a camera underneath its neck area, facing towards the ground, and a camera in the pelvis region, facing towards the ground as well. It has a small (105.5 x 67.2 x 3 mm screen; 95.04 x 53.86 mm actual screen size) screen which it can use to display text, animations, or other information that can be programmed. It has a microphone array in the chest area, and it features RGB lights in the chest cavity, and the head sides, for which the color can be controlled (by default used to indicate current robot state; Blue: Normal conditions, Red: Low Battery, Green: Charging).

It has a tablet that it can connect to via Bluetooth, which can be used to control its motions manually, activate certain functions Agibot created (some are in Beta), and perform some control over the robot without any need for development (It should be noted that for development, much of the tablet’s functionality must be deactivated).

It has 40 degrees of freedom, and is able to perform full arm and head motions. It can turn its head 120 degrees, and is able to move its arms fully.

It has a 360 degree LiDAR sensor that can scan upwards of 40 meters in poor conditions (10% reflectivity of surface) and 70 meters in good conditions (80% reflectivity of surface). The robot can use this LiDAR to map the space around it and it is crucial during locomotion that it scans the space around it for safety. The LiDAR is also used as a safety feature, where the robot will refuse to perform an action if an object is too close to the robot. This can be overridden, but we strongly recommend against this. The robot will hold whatever position it is currently in if an object appears near it and it doesn’t detect a collision.

# Section II – Hardware:

The A2 Ultra features 2 PCs, both of which perform different purposes. It is critical that the development of anything should be done on PC2, and PC1 should go untouched by developers for anything except making calls towards it and its functions or for connections to PC2. PC1 is the motor control/exterior PC. It features the motor controller, Network card, Sensor readings, and Screen configurations, all of which allow the robot to interact with the real world, and change how the robot interacts with it. PC2 holds the secondary development board. It is on the NVIDIA Jetson Orin AGX embedded computing device, which holds an NVIDIA configured Ubuntu 22.04 Version, named Jetpack 6.0. This is the standard, stable version of Jetpack, and this should go untouched. PC2 is the more powerful PC, which allows for development of compute-intensive operations, and general development.

The following will give an introduction into how the robot is divided, and how the different PCs work.

## PC 1:

PC1 holds the x86 computer, which acts as the motor controller, Network card, etc. Users are able to call to it to read sensor data, motor information and controls, and change the screen’s image or text. PC1 is low storage (a few GBs), and is only meant to run operations that either read from, or send commands to the PC. This way, it can perform critical tasks (reading and performing motor controls) entirely seperately from the main computer, meaining it can be near instant in performing tasks and the chance for unsafe situations is drastically decreased.

## PC 2:

PC2 is the NVIDIA Jetson Orin AGX, a powerful microcomputer, which allows for development of more compute intensive operations. It has 64GB of RAM, a set of ARM Cortex-A78AE CPUs, which has a max MHz rate of 2201.6001. It has 12 CPUs, and 4 Cores per cluster, and operates oin Little Endian with 32 and 64 bit op modes. It uses an NVIDIA Ampere architecture GPU with 2048 NVIDIA CUDA cores and 64 Tensor cores, allowing it to perform really well on even compute-intensive Transformer architectures.

This computer is very good for any development of software, as it allows the robot to perform many computations simultaneously without issues. Because the robot is on NVIDIA hardware, it is able to utilize TensorRT effectively, and as such can allow people to develop powerful AI models and run them entirelty locally on the machine.

This is also where the camera and LiDAR sensor data come to.

This PC can be programmed on safely, and it can also send commands to PC1 to perform actions.

## A2 – Cameras and LiDAR:

The A2 uses a powerful 360° LiDAR sensor (Livox Mid-360) in order to get a mostly-accurate depiction of the space around the robot. It is mounted in the “neck” area, where it is able to get a downward facing view of the surrounding space, allowing for a minimal amount of blind spots within the robot (there are a few, mentioned in a later section). It has a range of 40m in bad conditions (10% reflectivity of object), and about 70m in more ideal conditions (80% reflectivity), which makes it great for giving us the details of surrounding spaces.

It also features 3 chest cameras. On its chest cavity, it has 2 (left and right) fisheye cameras, which can help it see partially to the side, and it holds (center) a RGB camera, which has a more powerful image than any of the other cameras. It also has a camera in the “neck” area, which is aimed at a ~45° angle downwards, as well as a camera in its “groin” region, also with the ~45° angle downward.

## Warnings and Notices:

One thing to look out for when you develop is which PC you are currently on.

Running the command “whoami” will return

which is not a useful result. Instead, run hostname -I (can also just use hostname, although hostname -I is better in my opinion). This will return a list of addresses the current PC will apply itself to.

In this case, the user is in PC1, as one of the possible listed addresses is 192.168.100.100. For PC2, it will return 192.168.100.110. In PC1, you can also find the WiFi connection address (in this case, 10.198.9.48) which will allow others to connect to the PC without a physical cable.

Additionally, the LiDAR should, at no point in time, be disabled. Even if it may seem like extreme compute usage, the LiDAR works with several other systems (like gestures, vision, navigation, etc.) in order to ensure the robot is running safely. The LiDAR must also never be obscured or covered for long term sessions (unless doing testing that is approved by mentors), and must be given access to its surrounding view at all times.

Cameras, similarly, should not be turned off, covered, or otherwise defaced.

However, with explicit instructor/mentor consent, this can be arranged (turning off, not defacing).

Leaving the joints in a hold position other than by the side for extended periods of time can hurt the joint motor, and can lead to the robot overheating. Unless there is good reason, please avoid doing this. Consult the mentors on what you can/can’t do in regards to this for further details and for case-by-case bases.

# Section III – Software:

There is a lot of different types of software that can be used when developing the robot, both written by Comtrade and Agibot. The following sections will give quick introductions into the applications written by Comgrade and Agibot, respectively, and how they work together (and against each other in some cases).

## Software – Supervisor:

The Supervisor is Comtrade’s custom software developed for the Agibot A2 and X2 Ultra. It allows for configuration of different modules, including speech, gestures, movement, vision, etc. It allows users to utilize custom and Agibot software easily, and gives users greater control over the robot and its functionality. The Supervisor sits in the ‘humanoid-platform’ folder and can be started up using the command:

./run_robot_supervisor_v2.sh

which will launch the Supervisor. On the A2 Ultra, the Supervisor can be accessed using the address (while connected via Ethernet):

192.168.2.50:8070

For the X2 Ultra, it can be accessed using (while connected via Ethernet, otherwise use the WiFi address and the same port):

10.0.1.41:8080

Within the Supervisor, you’ll be able to find several different modules developed by Comtrade, all of which use different functionalities with the Robot.

#### Supervisor – Speech and Voice Agent:

The current Supervisor allows users to activate the voice agent and speech, which allows users to converse with the robot. Currently, we use a system of STT (Soniox) which then gets sent to an Azure OpenAI API call. A RAG call is made if necessary from the user’s question/prompt. The AI then returns a response, and uses TTS (Soniox/ElevenLabs; can choose) to provide the output in speech form. In this section of the Supervisor, there is opportunity to work on better prompting for the robot, greater speech recogntion/understanding, or determination of who is speaking to the robot. The user can also modify the robot’s persona, and make the robot speak in specific ways, giving users greater customization over the robot’s speech. The robot can also be given things to say without the need to activate the AI.

#### Supervisor – Computer Vision:

The robot has the ability to detect people, faces, and other objects. The current structure has several models, all of which can be turned on and off and changed within the code. The regular Vision Controller has access to the conversation, and controls the robot’s ability to converse. When the vision controller is active, users cannot manually activate the robot, but instead the robot will automatically start a conversation based on whether or not it detects a human being. The current controller uses a YOLOv26 model, with an option to use RT-DETR (custom – in development, so not perfect, YOLO is a safer option) for detecting humans.

The robot can also record and analyze human faces, memorizing names if offered the choice to do so (needs to be asked to remember someone’s name to save it). The robot uses a facenet architecture for facial recognition and memorization. It uses the human detection model to form a box around the human, then a CNN to detect faces within the box. Finally, the features are extracted (facial embeddings) and that is saved alongside a name in an SQLite database inside of the robot. The robot can then use these the next time it comes across a face, and use it.

#### Supervisor – Navigation and LiDAR:

The navigation module in the robot allows a user to give the robot checkpoints to walk towards, at which point it can perform actions, say given lines, or activate a custom script written by users.

In order to perform navigation, a few conditions need to be met:

- The robot must be localized → The robot’s pose, location, and map must be known to it before it can activate navigation. For help with doing this, ask the Mentor on standby at the A2, and they will localize the robot if need be. However, the robot should be localized when the event starts, assuming no one had turned it off after it localized.

- The robot must not be performing an animation, or be in idle mode. → This can be controlled using the “Arm” feature on the robot, which will activate it into a standby mode for movement and navigation (note that animations cannot be activated while the robot is in standby, but the given animations during navigation will work fine).

- The robot must not have anything in its nav points. If it does, it will stop and wait for the obstacle to move (did not add a feature to have it yell at user if they are in the spot, may add prior to hackathon).

The navigation uses a mixture of Agibot’s AimDK for movement, and some custom code for obstacle avoidance and movement through the map.

The Agibot AimDK needs a map to be formed in order to move around. The robot will need to move around the space, and use the LiDAR in order to map it so that it can have a global costmap of an area. The Supervisor also features a tab where users can view and read from the LiDAR sensor, giving them a 2D map live. You can use this code to generate functions using the LiDAR mapping that the Supervisor makes.

The scripts also feature the ability to manually control the robot, it’s arms, etc.

Users can generate arm point commands which are sent to the motor controller in PC1, allowing users to perform custom movements (for example, you can program a variable movement of arms to move towards grabbing something, shaking hands dynamically, etc.).

#### Supervisor – Miscellaneous Functions:

The Supervisor also has misc. functions that are mostly unused by the robot for now. These functions include, telling current time, weather, etc. These functions can be created on the side and attached into agent_main.py to allow greater depth of knowledge within the Supervisor and the robot’s conversational abilities.

## Software – Agibot AimDK:

AimDK is Agibot’s own software layer, and it is essential for anything using the motors without custom development. It is the controller of almost all systems in the robot, including walking, navigating, playing animations, reading sensors, building maps, etc. The Supervisor utilizes AimDK for a lot of the motor control, animation, and non-custom configurations (speech, vision, and parts of navigation don’t communicate with AimDK).

AimDK is organized as a set of services, each which offers a number of methods to use. A service is a topic area and a method is one thing you can ask the computer to perform. There are ~210 service definitions on the A2 Ultra. The largest group is motor control and sensor reading from hardware.

To reach AimDK, you simply make a plain HTTP call using JSON. It is integrated into the robot, so it can be used from any part of it. You can use curl, Python’s ‘requests’ library, JS’s ‘fetch’, etc. to call upon AimDK, allowing it to be programmed in whatever language users desire. All requests will follow the following shape:

http://<address>:<port>/rpc/aimdk.protocol.<Service_Name>/<Method_Name>

with a JSON body. For example, to get the current state of the system, you would run the curl command:

curl -s -X POST http://192.168.100.110:51056/rpc/aimdk.protocol.PncService/ActionGetState -H 'Content-type: application/json' -d '{"header":{}}'

You must send a header, even if it’s empty.

The services are split among the two PCs, where things like navigation, mapping, etc. are in PC2, and motor control and hardware control modules are in PC1. If you get a return of “resource not found” it will likely mean that the PC you are targeting is wrong. Specifics about locomotion and navigation are listed in the AimDK document.

AimDK also comes with a command-line tool called ‘aima’, which can be used to inspect parts of the robot, with ‘aima em doctor’ being a particularly useful main command which lets a user know what parts of the robot are active from AimDK. This will be useful for debuggning things like audio, navigastion, Teleoperations, etc. as AimDK tends to be greedy with resources and doesn’t share them. To activate certain modules with your own code, you may need to free up the resource and kill the AimDK module that is active at that time ‘aima em stop-app <app_name>’. By our default, the Supervisor will kill certain modules from AimDK, one such being ‘agent’.

#### AimDK – Warnings:

AimDK documentation is often not up-to-date on everything, and may have incorrect/innacurate information about specific modules. Ask a mentor about specific modules if there is an issue and we can help you figure out the issue. The current build of AimDK might not be up to date or the documentation could have details missing, so always double check.

AimDK also tends to be quiet if something goes wrong, returning a successful receiving instead of a successful action.

AimDK is very particular about how and when things run, particularly with Navigation. The robot will not run certain functions if the robot is not in an exact mode at the time the action is attempted. More details in the main document.

# Section IV – Locomotion and Motion Controls:

## General Information:

The Locomotion modules of the A2 Ultra have full control over the leg movements in the robot. This include walking, turning, balancing, etc.

Motion Controls, on the other hand, are the robot’s modules for moving the arms, torso, head. It is primarily used for gestures.

The A2 has rules for when these can be active at once, and with the exception for one case (mode), they cannot be active at the same time. The robot will fully reject commands sent to it if it is not in the correct mode, so this is something to beware. The robot has a set of modes that will accept specific commands, and silently reject the others.

With this in mind, you also cannot jump from one mode to another. The robot has a predefined pathing of modes that it will accept, and illegal jumps will be blocked (quietly ): ...). Within our Supervisor, the robot can be enabled into a 100% ready for walking mode via the “Arm” button in the Navigation sector, which can be activated even if navigation is not needed.

The robot will typically balance on its own, with a few exceptions. It has a balance controller in PC1 which is always running, and you will likely never have to worry about the robot balancing or falling by itself. For safety, keep the robot up on the gantry while performing tests, and once you are ready for a field test, you can remove the gantry.

Via AimDK, you never directly control the motors themselves, instead giving a command that includes the direction and speed you want. In the event of navigation, you may provide a coordinate point which the robot will go towards.

If PC2 ever dies (somehow) while navigation is occuring, or stops sending commands, PC1 will assume something went wrong, and stop the navigation. It will not finish navigation until PC2 responds to it again.

## How to use Locomotion via AimDK:

With AimDK, you have 3 levels of control when walking, which goes from highest level to lowest level control.

#### Level 1 – Map Nav:

You give the robot a destination, its current pose and location, and it will draw out a route and follow it. The Supervisor has this implemented into it.

#### Level 2 – Relative Moves:

This is how you control the robot outside of navigation. You can send it instructions which amount to “walk forward by x meters”. This can be used for scripted behaviors, general control. The Supervisor does not have a manual control function within it, but the scripts do have a function for this.

#### Level 3 – Velocity Commands:

You can send continuous commands of forward, sideways, and rotational speed to PC1. This gives greater control over movements, and can be used for custom planners, movement, etc. At this layer, however, there is no obstacle reaction, meaning the robot will effectively become a moving 70kg brick. When operating at this mode, be careful for people in path, and have an engineer on safety lookout for people in the robot’s path, especially if the robot is off the gantry.

## How to use Motion Controls via AimDK:

AimDK can be used for animations/gestures to be played. The A2 Ultra comes with 133 animations by default, and custom ones can be added via LinkCraft, or by recording your own joint movements.

## How Motion Controls work:

Animations on the robot are recorded motion files which are played by a dedicated player which streams to the joints. As mentioned above, there are 133 animations by default. Comtrade has tested and verified 20 as safe and good to use (listed in the Supervisor). The others work, but are more dangerous to work with. The animations can manually be played via the Supervisor’s manual speech section.

Here, you can manually test out the animations, and play them alongside predefined speech for the robot. This will use the defined speech service the robot is using. The animations not in the “safe” category are lenghtier, have more sweeping motions, and have a genrally larger chance of hurting someone. Be careful before launching the animations in this section.

Be careful when performing custom animations, either in your own manipulation or via LinkCraft, as the robot can collide with itself. Typically with animations, the robot will not stop itself from hitting itself (it might hate its existance, we don’t know yet :D), but this can result in the robot damaging itself. As such, please be careful when doing this, and do not perform actions that may hit the chest, or camera areas.

As mentiond before above, holding the joint in a single position can be bad for the motor, as they heat up. Whenever you don’t need it, return joints to the default netural pose, as that is the least power-intensive pose, and will allow the robot to safely cool itself.

## Safety and Dangers:

As mentioned before, think of this robot as a walking brick. It has safety features baked into it, but this isn’t guarenteed, especially if you’re using the velocity controller. As such, always be careful when running this robot, as it doesn’t have sensors in individual sections for detecting force or contact. If it kicks someone, or steps on them, there is a chance it won’t notice, and that’ll hurt pretty bad.

The robot’s motor controller and react modules live on PC1. PC1 is unobstructed as such, and has the fastest reaction time by default. The reaction time is very fast but not infinitely fast. PC2 is the slower layer, ironically, in which all the thinking, and sensor reading lives. As such, there is a moment between the latest LiDAR reading and the motor controller receiving it that the robot might miss. It is a small window, but one nonetheless. The LiDAR has no control over the motors, and as such it won’t perform anything. The LiDAR just draws a map of the surroundings and sends that data to PC2, which then uses it for its own purposes.

Additionally, the LiDAR has blind spots. The worst ones are behind the backpack, and near the arms, where the shoulders will obscure small objects/people. As such, be careful when moving around the robot, even when it is near.

Glass is another weakness for the robot. The robot’s LiDAR cannot see glass consistently. We at Comtrade have put in some features to find glass and counteract this issue, but it isn’t a guarentee, unfortunately. When moving near glass, keep someone near the robot’s backside (safest side to operate near the robot), keeping a hand near the handle on its backpack. When running motor tests, also contact the mentor on duty for overwatch.

# Section V – Emergency and Safety Features:

The robot has multiple safety features within it. There’s greater details in the safety guide and videos, but here are some of the more major safety features of the robot:

## E-Stop:

The robot comes with an Emergency Stop button. This button connects to the robot directly via a receiver in the neck area, and connects directly to the Motor Controller, bypassing any and all other commands. Once pressed, it will force the motors to shut down, and the robot will fall into a collapsed state. This remote will be in the hands of one of the mentors, and will be used in cases of extreme emergency.

## LiDAR Safety:

In the event that a person, object or anything detectable is too close to the robot, it will refuse to perform any form of animation, or non-locomotion movement it is given. The LiDAR runs 24/7 and captures data at all times, giving the robot an accurate depiciton of its environments. In the event that something is too close mid-animation, the robot will stop the animation and hold the position, until the object moves out of the way.

During navigation (Agibot’s and Comtrade’s), the robot will perform a stop in the event it detects something in the path. If an obstacle is 2.5 meters away, the robot will detect it and attempt to move around it, changing its course. If an object is <= 1.2 meter away, then the robot will perform a full stop, waiting a few seconds before trying to reroute. During reroute, it will slow its pace drastically, in an attempt not to hit something. The LiDAR keeps track of people and moving objects during this time, so something appearing in front of the robot suddenly will warrant a change in behavior.

## Supervisor Safety Features:

The Supervisor also has some safety features it imposes on top of the Robot’s own. During runtime of navigation, users can manually turn the robot’s navigation module off. This is near instant, an14 ptd won’t kill all motors.

Additionally, there is increased safety around glass areas. Due to the poor reflectivity of glass, the robot’s LiDAR system will often be confused when traversing around it. For the case of glass, we have added extra steps of safety, so that the robot doesn’t accidentally hit glass; however, this is still a Beta feature, so it isn;’t perfect.

There is also a feature called “Arm” in the robot’s navigation menu. This feature will allow users to “Arm” a robot, making it prepared for movement for navigation at any time. This also makes the robot unable to animate, so that it does not accidentally hit someone during its idle animation.