# AimDK/AimRT X2 Ultra Motor Control/Locomotion Docs

Comtrade System Integration – AI/Robotics Team

Contents

The Three Computers:	3

PC2 Hardware:	3

Network Layout:	4

Checking What Is Running:	4

The Two Control Surfaces:	5

Transports and Ports:	5

ROS 2 Environment:	6

Motor Controls:	6

The Joint Model:	6

Legs (12 joints):	6

Waist (3 joints):	7

Arms (14 joints):	7

Head (2 joints):	7

Reading Joint State:	8

Direct Joint Control:	8

Action Modes and the Three Gates:	8

Gate 1 – Input Source and Priority:	9

Gate 2 – Action Mode:	9

Gate 3 – Joint Limits:	10

Locomotion:	10

Level 1 – Map Navigation (PncService):	10

Level 2 – Preset and Relative Moves:	11

Level 3 – Velocity Commands:	11

Preset Motions (Gestures):	12

LiDAR-Based Mapping and Localisation:	13

Building a Map:	13

Relocalisation:	14

Stored Maps:	14

Navigation Configuration:	14

Worked Examples on the Robot:	15

Debugging Codes:	16

Summary of Differences from the A2 Ultra:	16

Potential Issues/Things we will need to Address:	17

Missing Safety Distance Parameters:	17

Glass:	17

Blind Spots:	17

Velocity Control Has No Guard Rail:	17

Documentation and Version Drift:	17

Supervisor Navigation Parity:	18

References:	18

The Agibot X2 Ultra runs AimRT in the same way the A2 Ultra does, but the surface that developers actually talk to is substantially different. On the A2, nearly everything was reachable as an HTTP-RPC call under /rpc/aimdk.protocol.<Service>/<Method>, and the SDK was effectively a catalogue of ~210 such service definitions. On the X2, AimDK ships as a ROS 2 interface package (aimdk_msgs), and motor control, locomotion, joint access and gestures are all done through ROS 2 topics and services. HTTP-RPC survives on the X2, but only for the navigation planner.

This document should be read alongside the A2 AimRT/AimDK document, not as a replacement for it. Do not assume that an endpoint which works on the A2 works here. Several of them do not exist at all on the X2, and a few exist under the same name on a different port. Where this document contradicts the A2 document, the difference is intentional and has been verified on the robot.

Everything below was verified on the physical X2 (PC2, 10.0.1.41) on September 22nd, 2026, against AimDK v1.0.0-g3a7e221. Items that could not be verified are called out explicitly rather than carried over from the A2 document on assumption.

# The Three Computers:

The single largest structural difference from the A2 is that the X2 has three computers, not two. The A2's split of “PC1 does motors and the outside world, PC2 does everything else” does not describe this robot.

| Unit | Role | Address | Use for custom code? |
| --- | --- | --- | --- |
| PC1 | Motion Control Computing Unit | not exposed to PC2 | No – vendor low-level work only |
| PC2 | Development Computing Unit | 10.0.1.41 | Yes – this is the default |
| PC3 | Interaction Computing Unit | 10.0.1.42 | Audio/voice device work only |

PC2 is where AimDK, the ROS 2 stack, the navigation modules and our own Supervisor live, and it is where essentially all of our development happens. PC3 exists because the interaction hardware (microphone array, speakers) is owned by a separate unit; if you need direct ALSA/PulseAudio device access, that work belongs on PC3 behind a small daemon, not on PC2.

## PC2 Hardware:

PC2 is an NVIDIA Jetson Orin NX (16GB), not the Orin AGX 64GB that the A2 carries. This is a real reduction in headroom and it matters when porting anything compute-heavy across from the A2.

| Property | X2 Ultra (PC2) | A2 Ultra (PC2) |
| --- | --- | --- |
| Module | Jetson Orin NX 16GB | Jetson Orin AGX 64GB |
| RAM | 16 GB (≈15 GB usable) | 64 GB |
| CPU | 8× ARM Cortex-A78AE | 12× ARM Cortex-A78AE |

| Property | X2 Ultra (PC2) | A2 Ultra (PC2) |
| --- | --- | --- |
| CPU max clock | 1984.0 MHz | 2201.6 MHz |
| Cores per cluster | 4 | 4 |
| L4T / JetPack | R36.4.3 (JetPack 6.2) | JetPack 6.0 |
| CUDA | 12.6 | – |
| Endianness / op modes | Little Endian, 32 & 64 bit | Little Endian, 32 & 64 bit |

NOTE 1: Budget roughly a quarter of the A2's RAM and about half its GPU capacity. Models that ran comfortably on the A2 may need quantising, batching changes or TensorRT conversion before they will run here.

## Network Layout:

PC2 presents several interfaces, and knowing which is which saves a lot of confusion. Run:

ip -br addr

On the X2 this returns the following (addresses as configured on our unit):

| Interface | Address | What it is |
| --- | --- | --- |
| develop0 | 10.0.1.41/24 | Inter-PC development network (PC2 ↔ PC3) |
| sensor0 | 10.11.1.1/24 | Sensor network; LiDAR answers on 10.11.1.100 |
| wifi0 | 10.198.0.218/20 | WiFi; use for remote access to the Supervisor |
| ssh0@develop0 | 10.0.200.41/24 | SSH VLAN on top of develop0 |
| docker0 | 172.17.0.1/16 | Local Docker bridge |
| can0 | down | CAN bus, not in use |

NOTE 2: The A2 trick of identifying the machine by looking for 192.168.100.100 (PC1) or 192.168.100.110 (PC2) does not work on the X2. That address range does not exist here. On the X2, look for 10.0.1.41 to confirm you are on PC2 and 10.0.1.42 for PC3.

NOTE 3: 'whoami' is as useless here as it was on the A2, but for a different reason – it returns 'run' on PC2 rather than 'agi'. 'hostname' returns 'agi' on PC2, which is actively misleading. Use 'hostname -I' or 'ip -br addr' and read the address.

# Checking What Is Running:

As on the A2, the first command to reach for when something has stopped working is:

aima em doctor

which lists every EM-managed application, its PID, start time, EM state and actual state. The X2's application list overlaps only partially with the A2's, so do not use the A2 document's screenshot as a reference for what “normal” looks like here. A healthy X2 shows the following applications:

| Application | Roughly what it does |
| --- | --- |
| pnc | Vectorflux – planning and navigation control |
| slam | SLAM / localisation |
| map_manager | Map storage and retrieval |

| Application | Roughly what it does |
| --- | --- |
| perception | Obstacle and object perception |
| auto_integrated | Integrated control / high-level command routing |
| hal_sensor_orin | Sensor HAL (LiDAR, IMU, cameras) on the Orin |
| orbbec_camera | Orbbec camera driver |
| hds_master / hds_slave | Health diagnostic system |
| dm / drp / ec / atop | Device, data-routing, execution and monitoring services |
| cobridge | Bridge / external connectivity |
| housekeeper, ota | Maintenance and updates |

NOTE 4: Applications that exist on the A2 and are absent on the X2 include 'gateway', 'agent', 'hal_audio', 'hal_lidar', 'hal_d415', 'manipulator', 'task_engine', 'resource_manager', 'agivslam' and 'iox_roudi'. If you are following an A2 procedure that tells you to stop or restart one of these, it will simply not be there.

To free a resource that AimDK is holding, the same EM commands work as on the A2:

aima em stop-app <app_name> aima em start-app <app_name>

aima em reset-app <app_name>

NOTE 5: 'aima topic list' and 'aima node list' depend on the health diagnostic MonitorService on 127.0.0.1:50587. At time of writing that service was not listening, and both commands fail with a connection-refused error rather than an empty list. This is a diagnostic gap, not necessarily a fault in the module you are investigating.

# The Two Control Surfaces:

Everything you can command on the X2 goes through one of two transports. Getting this distinction right at the start will save a great deal of time.

| Surface | Transport | Covers |
| --- | --- | --- |
| AimDK ROS 2 (aimdk_msgs) | ROS 2 topics and services | Action/mode changes, locomotion velocity, direct joint control, hands, preset motions, sensors, audio, LEDs, mapping commands |
| PncService | HTTP-RPC (JSON) on 127.0.0.1:53176 | Map navigation, relative/linear moves, spin, docking, task lifecycle |

The ROS 2 surface is the one that replaces the A2's McMotionService, McBaseService and McActionService. The HTTP-RPC surface is a survivor from the A2 and behaves almost identically, just on a different port.

## Transports and Ports:

| Type of Transport | Where to Send | Reason |
| --- | --- | --- |
| HTTP-RPC | PncService: 127.0.0.1:53176 | Navigation commands and queries |
| HTTP-RPC | Motion Controller: ${MC_IP}:56322 | Low-level MC queries (GetAction, GetLocomotionGait) |
| HTTP-RPC | Relocalisation: 127.0.0.1:50807 | StartGlobalRelocalization, StartFree |

| Type of Transport | Where to Send | Reason |
| --- | --- | --- |
| ROS 2 topics/services | aimdk_msgs interfaces | Motor control, sensors, gestures, audio |
| UDP | Motion Controller: ${MC_IP}:50041 | Locomotion velocity stream |

NOTE 6: The A2's gateway on port 51056 does not exist on the X2. There is no 'gateway' application in EM and nothing is listening on that port. Any A2 script pointed at 51056 will fail with a connection refused, not a 404, which makes it easy to misdiagnose as a network problem.

NOTE 7: MC_IP is injected into the navigation apps at launch by EM. It is not defined in any file under /agibot/software or /agibot/etc, and because 'pnc' runs as root its environment is not readable from a normal developer account. The motion controller did not answer on 10.0.1.40 or

10.0.1.42 from PC2 on ports 56322/56321. Resolving MC_IP is an open item – see the final section. In practice you should not need it: use the ROS 2 interfaces, which are routed for you.

## ROS 2 Environment:

Before importing rclpy or running any ros2 command against the robot stack, set:

source /opt/ros/humble/setup.bash export ROS_DOMAIN_ID=232 export ROS_LOCALHOST_ONLY=0

export FASTRTPS_DEFAULT_PROFILES_FILE=\

/agibot/software/entry/cfg/ros_dds_configuration.xml

NOTE 8: The FastDDS profile shipped on the robot pins an interfaceWhiteList to 10.0.1.41 (develop0). DDS discovery will therefore not cross to WiFi or any other interface. If you are trying to reach AimDK topics from a laptop over WiFi and seeing nothing at all, this whitelist is why. The file is host-specific: do not copy it to PC3 unchanged.

NOTE    9:    Our    Supervisor's    camera    documentation    points    at

/agibot/software/v0/entry/bin/cfg/ros_dds_configuration.xml. That path does not exist on the X2. The correct path is /agibot/software/entry/cfg/ros_dds_configuration.xml. This needs fixing in docs/agibot/camera_handling.md.

NOTE 10: ROS_DOMAIN_ID is not pinned in any nav config on disk; it comes from the launch environment. At time of writing, scanning domains 0, 100, 231 and 232 from PC2 returned only /parameter_events and /rosout, i.e. no AimDK topics were discoverable. Confirm the domain and that the MC stack is actually up before concluding your own code is at fault.

# Motor Controls:

## The Joint Model:

The X2 exposes 31 controllable joints across four areas, plus hands. This is a different body from the A2, which had 26 joints, a fixed head and no waist. In particular the X2 has a 3-DOF waist and a 2-DOF head, and the leg chain uses a conventional knee/ankle arrangement rather than the A2's tarsus/toe joints.

Joints are addressed by name, not by index. The A2's idx 1–26 scheme does not apply. Each joint has a lower and upper limit in radians and a default stiffness (kp) and damping (kd).

### Legs (12 joints):

| Joint name | Lower | Upper | kp | kd |
| --- | --- | --- | --- | --- |
| left_hip_pitch_joint | -2.704 | 2.556 | 40.0 | 4.0 |
| left_hip_roll_joint | -0.235 | 2.906 | 40.0 | 4.0 |
| left_hip_yaw_joint | -1.684 | 3.430 | 30.0 | 3.0 |
| left_knee_joint | 0.0000 | 2.4073 | 80.0 | 8.0 |
| left_ankle_pitch_joint | -0.803 | 0.453 | 40.0 | 4.0 |
| left_ankle_roll_joint | -0.2625 | 0.2625 | 20.0 | 2.0 |
| right_hip_pitch_joint | -2.704 | 2.556 | 40.0 | 4.0 |
| right_hip_roll_joint | -2.906 | 0.235 | 40.0 | 4.0 |
| right_hip_yaw_joint | -3.430 | 1.684 | 30.0 | 3.0 |
| right_knee_joint | 0.0000 | 2.4073 | 80.0 | 8.0 |
| right_ankle_pitch_joint | -0.803 | 0.453 | 40.0 | 4.0 |
| right_ankle_roll_joint | -0.2625 | 0.2625 | 20.0 | 2.0 |

### Waist (3 joints):

| Joint name | Lower | Upper | kp | kd |
| --- | --- | --- | --- | --- |
| waist_yaw_joint | -3.43 | 2.382 | 20.0 | 4.0 |
| waist_pitch_joint | -0.314 | 0.314 | 20.0 | 4.0 |
| waist_roll_joint | -0.488 | 0.488 | 20.0 | 4.0 |

### Arms (14 joints):

| Joint name | Lower | Upper | kp | kd |
| --- | --- | --- | --- | --- |
| left_shoulder_pitch_joint | -3.08 | 2.04 | 20.0 | 2.0 |
| left_shoulder_roll_joint | -0.061 | 2.993 | 20.0 | 2.0 |
| left_shoulder_yaw_joint | -2.556 | 2.556 | 20.0 | 2.0 |
| left_elbow_joint | -2.3556 | 0.0 | 20.0 | 2.0 |
| left_wrist_yaw_joint | -2.556 | 2.556 | 20.0 | 2.0 |
| left_wrist_pitch_joint | -0.558 | 0.558 | 20.0 | 2.0 |
| left_wrist_roll_joint | -1.571 | 0.724 | 20.0 | 2.0 |
| right_shoulder_pitch_joint | -3.08 | 2.04 | 20.0 | 2.0 |
| right_shoulder_roll_joint | -2.993 | 0.061 | 20.0 | 2.0 |
| right_shoulder_yaw_joint | -2.556 | 2.556 | 20.0 | 2.0 |
| right_elbow_joint | -2.3556 | 0.0000 | 20.0 | 2.0 |
| right_wrist_yaw_joint | -2.556 | 2.556 | 20.0 | 2.0 |
| right_wrist_pitch_joint | -0.558 | 0.558 | 20.0 | 2.0 |
| right_wrist_roll_joint | -0.724 | 1.571 | 20.0 | 2.0 |

### Head (2 joints):

| Joint name | Lower | Upper | kp | kd |
| --- | --- | --- | --- | --- |
| head_yaw_joint | -0.366 | 0.366 | 20.0 | 2.0 |
| head_pitch_joint | -0.3838 | 0.3838 | 20.0 | 2.0 |

NOTE 11: The head range is small – roughly ±21° yaw and ±22° pitch. The A2 document's claim of 120° of head rotation does not transfer to the X2, and should be re-checked for the A2 as well, since the A2's own AimDK document describes the head as fixed on the with_casing variant.

NOTE 12: Unlike the A2, the elbow limits include 0.0, so a fully straight elbow is representable on the X2. Do not carry over the A2's “the elbow can never be 100% straight” rule.

## Reading Joint State:

Two ways. As a one-shot service call, which returns the four areas separately:

/aimdk_5Fmsgs/srv/GetAllJointState

The response contains head_joints, arm_joints, waist_joints and leg_joints, each an array of JointState. For continuous reading, subscribe to the per-area state topics:

/aima/hal/joint/leg/state

/aima/hal/joint/arm/state

/aima/hal/joint/waist/state

/aima/hal/joint/head/state

/aima/hal/joint/hand/state

NOTE 13: State topics are published BEST_EFFORT. If you subscribe with the default RELIABLE QoS you will silently receive nothing. This is the single most common mistake when first reading sensors on this robot.

## Direct Joint Control:

Joint commands are published as a JointCommandArray of JointCommand messages to the matching command topic:

/aima/hal/joint/leg/command

/aima/hal/joint/arm/command

/aima/hal/joint/waist/command

/aima/hal/joint/head/command

/aima/hal/joint/hand/command

Each JointCommand carries:

| Field | Type | Meaning |
| --- | --- | --- |
| name | string | Joint name from the tables above |
| position | float64 | Target position, radians |
| velocity | float64 | Target velocity |
| effort | float64 | Feed-forward torque; normally 0.0 |
| stiffness | float64 | Position gain (kp) |
| damping | float64 | Velocity gain (kd) |

Command topics must be published RELIABLE, on a 2 ms control period (500 Hz). The vendor example uses the Ruckig trajectory planner to generate smooth position, velocity and acceleration profiles between the current and target state rather than stepping the target directly. Do the same: a step change in target position at this control rate is a good way to damage a joint.

NOTE 14: This is the lowest level of control on the robot and there is no obstacle reaction, no self-collision check and no balance protection at this layer. Keep the robot on the gantry, and do not send leg commands while the robot is standing on its own unless you know exactly what you are doing.

# Action Modes and the Three Gates:

The A2 had three gates – control source, action mode and planner bounds. The X2 has an equivalent set of three, but only the middle one resembles its A2 counterpart.

## Gate 1 – Input Source and Priority:

Before the motion controller will accept anything from you, you must register an input source.

This replaces the A2's ControlSource_SAFE header field. Call:

/aimdk_5Fmsgs/srv/SetMcInputSource

with a SetMcInputSource.Request containing:

| Field | Value | Meaning |
| --- | --- | --- |
| action.value | 1001 | INPUTACTION_ADD (also: 1002 MODIFY, 1003 DELETE, 2001 ENABLE, 2002 DISABLE) |
| input_source.name | e.g. “node” | Source name; known names are rc, vr, app_proxy, interaction, pnc |
| input_source.priority | 0–100 | Higher wins. The vendor examples use 40. |
| input_source.timeout | 1000 | Milliseconds before the registration lapses |

NOTE 15: The timeout is a deadman switch, not a one-off handshake. If you stop publishing, your source lapses after the timeout and control reverts. This is a safety feature and you should not raise the timeout to work around it.

NOTE 16: Priority matters. Registering at a priority above the teleoperation remote or the built-in interaction module means you can take control away from them. Stay at or below 40 unless you have a specific reason and a mentor present.

## Gate 2 – Action Mode:

The robot will silently reject commands that do not suit its current mode, exactly as on the A2. What has changed is how modes are named. The A2's composite string form – McAction_RL_LOCOMOTION_ARM_EXT_PLANNING_MOVE, encoding leg behaviour and arm interface in one identifier – does not exist on the X2. The X2 uses a flat integer enumeration:

| Mode | Value | What it does |
| --- | --- | --- |
| PASSIVE_DEFAULT | 1 | Passive; joints at zero torque |
| SOFT_EMERGENCY_STOP | 2 | Software emergency stop |
| DAMPING_DEFAULT | 3 | Damping mode |
| ZERO_TORQUE_DEFAULT | 4 | Zero torque |

| Mode | Value | What it does |
| --- | --- | --- |
| JOINT_DEFAULT | 100 | Position-control stand; joints locked |
| JOINT_FREEZE | 101 | Position-control joint freeze |
| STAND_DEFAULT | 200 | Force-control stand, auto-balancing |
| STAND_BODY_CONTROL | 201 | Stand plus body motion |
| LOCOMOTION_DEFAULT | 300 | Walking |
| RUN_DEFAULT | 301 | Running |
| LOCOMOTION_STEP | 302 | Off-road / stepping mode |
| VR_REMOTE_CONTROLLER | 400 | VR teleoperation |
| SIT_DOWN_DEFAULT | 2000 | Skill: sit down |
| CROUCH_DOWN_DEFAULT | 2002 | Skill: crouch |
| LIE_DOWN_DEFAULT | 2004 | Skill: lie down |
| STAND_UP_DEFAULT | 2005 | Skill: stand up from lying |
| ASCEND_STAIRS | 2006 | Skill: ascend stairs |
| DESCEND_STAIRS | 2008 | Skill: descend stairs |

To change mode, call:

/aimdk_5Fmsgs/srv/SetMcAction

The request takes a header, a source string and an McActionCommand. In the vendor example the mode is selected by name via command.action_desc (for example “STAND_DEFAULT”) rather than by the integer. To read the current mode, call GetMcAction, which returns current_action, action_desc and a status of IDLE (0), RUNNING (100) or TRANSITION (200).

The normal ladder up to walking is:

PASSIVE_DEFAULT → DAMPING_DEFAULT → JOINT_DEFAULT → STAND_DEFAULT → LOCOMOTION_DEFAULT

NOTE 17: As on the A2, you cannot jump arbitrarily between modes, and illegal transitions are rejected quietly. Step through the ladder and poll GetMcAction until status leaves TRANSITION before sending the next request.

NOTE 18: PASSIVE_DEFAULT and ZERO_TORQUE_DEFAULT drop the motors. Do not enter either without the gantry. This is the X2 equivalent of the A2's PASSIVE_UPPER_BODY_* warning.

NOTE 19: SOFT_EMERGENCY_STOP is a software stop and is not a substitute for the physical E-Stop. It goes through the same service call path as every other mode change, so if that path is wedged, the software stop is wedged with it.

## Gate 3 – Joint Limits:

Every joint must be within the limits listed in the joint model tables. The A2's distinction between planner bounds and URDF bounds does not appear on the X2; the limits in the vendor robot model are what the controller enforces. As on the A2, an out-of-range value causes the goal to be discarded rather than clamped.

NOTE 20: AimDK on the X2 remains as quiet about failures as it was on the A2. A service call returning successfully means the request was received, not that the robot did anything. Always verify by reading state back – GetMcAction for modes, GetAllJointState or the state topics for joints.

# Locomotion:

As on the A2 there are three levels of control, from highest to lowest. The boundaries are in roughly the same places, but the transports differ.

## Level 1 – Map Navigation (PncService):

This is the survivor from the A2, and the one place where the familiar HTTP-RPC shape still applies:

http://127.0.0.1:53176/rpc/aimdk.protocol.PncService/<Method>

with a JSON body, and a header that must be present even when empty. To read the current navigation state:

curl -s -X POST \ http://127.0.0.1:53176/rpc/aimdk.protocol.PncService/ActionGetState \

-H 'Content-Type: application/json' -d '{"header":{}}'

which returns, on an idle robot:

{"task_id":"0","state":"PncServiceState_IDLE"}

The methods present in the installed build are:

| Method | Inputs | Use |
| --- | --- | --- |
| PlanningNaviToGoal | map_id, target_id, guide_line_id | Full planner to a saved navigation point |
| PlanningNaviToPose2D | SE2Pose | Planner to an arbitrary coordinate |
| DirectNaviToRelative / DirectNaviToRelativeGoal | SE2Pose | Straight to a relative pose |
| LinearNaviToGoal / ToPose2D / ToRelative | – | Linear motion |
| PreciseNaviToGoal | target_id, pose_offset | Fine docking |
| MoveForward | angle, distance | Open-loop |
| SpinTurn / SpinTurnAndMoveForward | angle, [distance] | Rotation |
| ActionCancel / Pause / Resume / GetState | task_id | Lifecycle |
| SetRelocationStatus | – | Relocalisation state |
| CalculateNaviTime / AutoExplore / NoMapFollow | – | Estimation and exploration |
| VoiceControl | – | Voice-driven navigation entry point |

NOTE 21: Methods present on the A2 but not found in this build: MobileNaviToGoal / MobileNaviToPose2D, SetNaviParams and RecordCurrentPose. The loss of SetNaviParams matters – on the A2 that was how you tuned navigation at runtime without a restart. On the X2 you must edit the config and restart the app.

NOTE 22: SE2Pose is {position: {x, y}, angle} in metres and radians, as on the A2.

The task lifecycle is unchanged from the A2:

send task_id → poll ActionGetState → IDLE / RUNNING / PAUSED / SUCCESS / FAILED

## Level 2 – Preset and Relative Moves:

MoveForward, SpinTurn and the DirectNaviToRelative family give you scripted, bounded movements without writing a planner. These are the safest way to move the robot under your own code, because obstacle handling still applies.

## Level 3 – Velocity Commands:

The lowest level of locomotion control. Publish McLocomotionVelocity messages to:

/aima/mc/locomotion/velocity

The message carries a header, a source string matching your registered input source, and:

| Field | Units | Sign convention | Accepted range |
| --- | --- | --- | --- |
| forward_velocity | m/s | + forward, − backward | 0, or ±0.2 – 1.0 |
| lateral_velocity | m/s | + left, − right | 0, or ±0.2 – 1.0 |
| angular_velocity | rad/s | + left, − right | 0, or ±0.1 – 1.0 |

The sequence is:

- Register an input source (Gate 1) and check it succeeded.

- Set the robot to LOCOMOTION_DEFAULT (Gate 2) and wait for the transition to finish.

- Publish McLocomotionVelocity on a 50 Hz timer (0.02 s).

- Publish zeroes and stop the timer when done; handle SIGINT/SIGTERM to zero the velocity on exit.

NOTE 23: The motion controller has a deadband. Values between 0 and the minimum in the table above will not produce movement, and the vendor example raises an error rather than sending them. Do not interpret “nothing happened” at 0.1 m/s as a broken pipeline.

NOTE 24: As on the A2, there is no obstacle reaction at this layer. The robot becomes a walking mass that will go where you tell it. Keep an engineer on safety watch with eyes on the path, keep the robot on the gantry for first runs, and always install a signal handler that zeroes the velocity – if your process dies with a non-zero velocity latched, the robot keeps walking until the input-source timeout lapses.

A keyboard teleoperation example is provided at py_examples/keyboard.py and is the quickest way to confirm the whole chain works: W/S forward and back, A/D strafe, Q/E turn, Space to stop, Esc to exit, stepping by 0.2 m/s and 0.1 rad/s.

# Preset Motions (Gestures):

The A2's model of 133 recorded animation files played by a dedicated player, of which Comtrade verified 20 as safe, does not apply to the X2. The X2 exposes a fixed set of firmware preset motions selected by integer ID through:

/aimdk_5Fmsgs/srv/SetMcPresetMotion

The request takes:

| Field | Meaning |
| --- | --- |
| header | RequestHeader with a timestamp |
| area | McControlArea bitmask – see below |
| motion | McPresetMotion value |
| interrupt | Whether to interrupt the motion currently playing |
| ani_path | Path to a custom animation, for LinkCraft resources |
| play_timestamp | UNIX time in ms to start; 0 means immediately |

The control area is a bitmask, so areas can be combined:

| Area | Value |
| --- | --- |
| NONE | 0 |
| LEFT_HAND | 1 |
| RIGHT_HAND | 2 |
| HEAD | 4 |
| WAIST | 8 |

The preset motions available are:

| ID | Motion | ID | Motion |
| --- | --- | --- | --- |
| 1001 | Raise hand | 3003 | Peace sign |
| 1002 | Wave hand | 3004 | Heart over head |
| 1003 | Shake hand | 3006 | Sad |
| 1004 | Flying kiss | 3007 | Light wave |
| 1008 | Clap hands | 3008 | Hug |
| 1009 | Fist bump | 3009 | Arms crossed |
| 1013 | Salute | 3010 | Chest wave |
| 2001 | Turn and wave | 3011 | Cheer |
| 3001 | Bow | 3012 | Low blow-kiss |
| 3002 | Thumbs up | 3013 / 3014 | Bass dance 1 / 2 |
| 3015 | High five | 3016 | Speaking gesture |
| 3018 | Photo pose | 3019 | Triple photo pose |
| 4001 | Nod head | 4002 | Shake head |

NOTE 25: A successful response means the motion was queued, not completed. Check response.header.code == 0, or a state of RUNNING, and use the returned task_id to follow it.

NOTE 26: As on the A2, the robot will not stop itself from colliding with itself during a custom animation. Be careful with anything supplied through ani_path, and avoid motions that sweep across the chest or head, where the cameras and LiDAR are.

NOTE 27: Our Supervisor's gesture catalogue is pinned to the IDs present in the locally installed AimDK. Do not copy motion IDs from newer AimDK documentation into the catalogue – IDs have moved between versions and a wrong ID will play the wrong motion rather than fail.

# LiDAR-Based Mapping and Localisation:

The X2 carries the same class of 360° LiDAR as the A2, but it is mounted in the chest, not the neck – the topic is /aima/hal/sensor/lidar_chest_front/lidar_pointcloud, with a matching IMU on the same prefix. The cameras have moved the other way, onto the head. Blind-spot behaviour therefore differs from the A2 and should be re-characterised rather than assumed.

## Building a Map:

Mapping is driven by publishing plain strings to the /integrated_command topic, handled by the auto_integrated application:

start_mapping

stop_mapping:<map_name>

Walk the robot around the space between the two commands, as on the A2. The vendor example is py_examples/slam.py.

## Relocalisation:

Relocalisation uses the same topic plus a pose:

- Publish start_relocalization:<timestamp_ms> to /integrated_command.

- Publish a geometry_msgs/Pose estimate to /relocalization_pose after a short delay.

- Subscribe to /slam/lidar_loc (nav_msgs/Odometry, BEST_EFFORT) to confirm convergence. There is also an HTTP-RPC route for this on 127.0.0.1:50807 (StartGlobalRelocalization,

StartFree). The vendor example is py_examples/relocate.py.

## Stored Maps:

To retrieve a stored map by name:

/aimdk_5Fmsgs/srv/GetStoredMapByName

The response returns code, map_path, map_id and a map_info block carrying resolution, width and height. Maps themselves are stored in an SQLite database in the same location as on the A2:

/agibot/data/var/MapManagerModule/map.db

The schema is:

| Table | Columns |
| --- | --- |
| map | map_id (PK), map_version, map_name, map_info, topo |
| current_map | map_id (PK) |
| db_version | version |

NOTE 28: map_version remains unreliable, as on the A2. Order by map_id. The A2 document's notes on the occupancy map – top-left origin with v pointing down, the three-colour PNG, the BGR/RGB discrepancy and the two different meanings of “resolution” – have not been re-verified on the X2 and should be treated as unconfirmed here.

# Navigation Configuration:

The navigation parameters live at a different path from the A2:

/agibot/software/nav/config/vectorflux_common/a2/app2.yaml

Note that the X2 loads the directory named a2 with the a2w behaviour tree; this is a vendor naming artefact and not a sign that you are editing the wrong robot's config. The active configuration is selected in /agibot/software/nav/config/vectorflux/vectorflux_app_cfg.yaml.

The values on the X2 differ from the ones recorded for the A2:

| Parameter | A2 documented | X2 actual |
| --- | --- | --- |
| footprint | 0.672 m (scalar) | 8-vertex polygon; plus circular_footprint r = 0.18 |
| inflation_radius | 0.8 | 0.65 (×3) and 1.0 (×1) |
| cost_scaling_factor | 5.5 | 8.0 |
| inflation_by_lateral_width | true | true |
| spin_footprint | 0.70 m | not present |
| stop_dist_normal | 1.2 m | not present |
| slowdown_dist_normal | 2.5 m | not present |
| wait_change_path_delay_ms | 2000 | not present |

NOTE 29: The four missing parameters matter. stop_dist_normal and slowdown_dist_normal are the values behind the A2 document's claim that the robot stops at 1.2 m and slows at 2.5 m. Neither parameter exists anywhere under /agibot/software on the X2. Do not repeat those distances in X2 material until the actual stopping behaviour has been measured on this robot.

NOTE 30: As on the A2, inflation_radius appears several times in the file and each instance affects a different part of the navigation stack. Search for all occurrences and record which one you changed and why.

NOTE 31: The config is read only at startup. Vectorflux runs as the app 'pnc'. To apply a change: 'aima em reset-app pnc', or 'aima em stop-app pnc' followed by 'aima em start-app pnc'.

NOTE 32: DO NOT EDIT THE NAVIGATION CONFIG WHILE THE ROBOT IS

STANDING ON ITS OWN. Restarting pnc in certain standing modes can drop the robot. Gantry first, every time.

# Worked Examples on the Robot:

AimDK ships runnable Python and C++ examples covering every interface in this document. They are the fastest way to confirm a path works before writing your own code, and they are authoritative in a way the documentation sometimes is not:

/agibot/software/aimdk/src/py_examples/py_examples/

| Example | Covers |
| --- | --- |
| set_mc_action.py / get_mc_action.py | Mode changes and reading current mode |
| set_mc_input_source.py / get_current_input_source.py | Gate 1 registration |
| mc_locomotion_velocity.py | Velocity control with input-source registration |
| keyboard.py | Keyboard teleoperation |
| motocontrol.py | Direct joint control with Ruckig planning |

| Example | Covers |
| --- | --- |
| preset_motion_client.py | Preset motions / gestures |
| hand_control.py / omnihand_control.py | Dexterous hand control |
| slam.py / relocate.py / get_map.py | Mapping, relocalisation, stored maps |
| echo_lidar_data.py / echo_imu_data.py | LiDAR and IMU reading |
| echo_camera_rgbd.py / echo_camera_stereo.py | Camera streams |
| play_tts.py / play_audio.py / play_lights.py | Interaction: speech, audio, LEDs |
| play_linkcraft.py | LinkCraft animation playback |

C++ equivalents of the same examples are under src/examples/src/.

# Debugging Codes:

| Symptom | Likely cause | Where to check |
| --- | --- | --- |
| Connection refused on 51056 | A2 gateway port; does not exist on X2 | Use 53176 for PncService |
| HTTP 404 on an A2 endpoint | Method not in this build | Check the PncService method table |
| Service call succeeds, nothing moves | Gate 1 or Gate 2 | GetCurrentInputSource, GetMcAction |
| Mode change silently ignored | Illegal transition | Step the ladder; poll for TRANSITION to clear |
| Joint goal discarded | Gate 3, out of range | Compare against the joint limit tables |
| Subscriber receives nothing | QoS mismatch | State topics are BEST_EFFORT, not RELIABLE |
| No topics visible at all | DDS interface whitelist or wrong domain | ros_dds_configuration.xml; ROS_DOMAIN_ID |
| 'aima topic list' fails | MonitorService on 50587 not running | aima em doctor; restart the monitor |
| Robot walks after script exits | Velocity latched, no signal handler | Zero velocity on SIGINT/SIGTERM |
| Service call times out intermittently | Known ROS remote-peer issue | Retry the call; vendor examples retry 8× |

NOTE 33: Every vendor example wraps its service calls in a retry loop of eight attempts with a

0.25 s timeout, commented “retry as remote peer is NOT handled well by ROS”. This is not defensive padding – single-shot calls do fail intermittently. Copy the pattern.

# Summary of Differences from the A2 Ultra:

| Area | A2 Ultra | X2 Ultra |
| --- | --- | --- |
| Computers | 2 (PC1 x86, PC2 Orin AGX) | 3 (PC1 motion, PC2 Orin NX, PC3 interaction) |
| PC2 module | Orin AGX, 64 GB | Orin NX, 16 GB |
| Internal addressing | 192.168.100.100 / .110 | 10.0.1.41 (PC2), 10.0.1.42 (PC3) |

| Area | A2 Ultra | X2 Ultra |
| --- | --- | --- |
| AimDK form | ~210 HTTP-RPC service definitions | ROS 2 package: 28 srv, 59 msg |
| Primary transport | HTTP-RPC + JSON | ROS 2 topics and services |
| Gateway port | 51056 | does not exist |
| PncService | 51056 | 53176, same URL shape |
| Mode naming | Composite string, leg + arm encoded | Flat integer enum |
| Joints | 26, addressed by index | 31, addressed by name, plus hands |
| Waist / head | No waist; head fixed | 3-DOF waist; 2-DOF head |
| Animations | 133 files, 20 verified safe | ~28 firmware presets by ID |
| LiDAR mounting | Neck | Chest |
| Cameras | 3 chest, 1 neck, 1 pelvis | Head: RGB, RGB-D, stereo pair, rear |
| Camera driver | RealSense (hal_d415) | Orbbec (orbbec_camera) |
| Runtime nav tuning | SetNaviParams | Not available; edit config and restart |
| Supervisor navigation | Checkpoints and “Arm” feature | Not implemented |
| Supervisor extras | – | VR teleop, dexterous hands, HAL audio |
| Map database | /agibot/data/var/ MapManagerModule/map.db | Same path and schema |
| ROS domain | 232 (default 231 is wrong) | 232 per Supervisor; not pinned on disk |

# Potential Issues/Things we will need to Address:

MC_IP is Unresolved:

The navigation stack reaches the motion controller through a ${MC_IP} variable that is injected at launch and is not recorded in any file on disk. Because pnc runs as root, its environment cannot be read from a developer account, and the motion controller did not answer on the addresses reachable from PC2. We should get the real value from Agibot and document it, because without it the debugging table entry “is the MC even up?” cannot be answered directly.

## Missing Safety Distance Parameters:

The X2 navigation config contains no stop_dist_normal, slowdown_dist_normal, spin_footprint or wait_change_path_delay_ms. Either the X2 uses different parameter names for the same behaviour, or these distances are compiled in. We need to find out which, and then measure the actual stop and slowdown distances rather than inheriting the A2's figures. Until that is done we should not publish stopping distances for the X2 in any operator-facing material.

## Glass:

The A2 documentation states that Comtrade added features to detect glass and add extra safety steps around it. No such code exists in the Supervisor on the X2 – the only LiDAR-touching component is the viewer. If a glass aid is added here, it should be described accurately: a visual aid that flags suspected glass in the LiDAR tab, which does not feed navigation and will not stop the robot. Anything stronger invites people to trust it with their safety.

## Blind Spots:

The A2's blind spots were characterised for a neck-mounted LiDAR. The X2's LiDAR is chest-mounted, so the blind-spot geometry is different and has not been mapped. This needs a deliberate characterisation pass – particularly low and close to the robot, where a chest mounting is likely to be worse than a neck mounting.

## Velocity Control Has No Guard Rail:

Level 3 velocity control has no obstacle reaction, and a crashed process leaves the last velocity latched until the input-source timeout expires. We should write a small supervised wrapper for this path that owns the input-source registration, enforces a watchdog and zeroes velocity on any abnormal exit, rather than having each developer reimplement the signal handling.

## Documentation and Version Drift:

Our own gesture documentation refers to AimDK 0.8.24 while the installed package reports v1.0.0-g3a7e221, and our camera documentation points at a DDS profile path that does not exist on this robot. Both should be corrected. More generally, as with the A2, the vendor documentation is not always current with the installed build – prefer the interface definitions under aimdk_msgs and the shipped examples over any prose, including this document.

## Supervisor Navigation Parity:

The Supervisor has no navigation module on the X2: no checkpoints, no “Arm” feature, no nav points. Everything in this document at Level 1 and Level 2 is currently reachable only by direct PncService calls. If we want operator-facing navigation on the X2 we will need to port that module, and the port is not trivial because the A2 version assumes AimDK endpoints that do not exist here.

# References:

Agibot X2 AimDK online documentation: https://x2-aimdk.agibot.com

Local AimDK documentation: /agibot/software/aimdk/ → sh ./run_docs.sh 5789, then http://127.0.0.1:5789

Interface definitions: /agibot/software/aimdk/src/aimdk_msgs/interface/robot/ Python examples: /agibot/software/aimdk/src/py_examples/py_examples/ C++ examples: /agibot/software/aimdk/src/examples/src/

X2 RL deploy controller: /agibot/software/aimdk/extra/x2-rl-deploy/

Navigation config: /agibot/software/nav/config/vectorflux_common/a2/app2.yaml Livox Mid-360 LiDAR specs: https://www.livoxtech.com/mid-360/downloads